# Reliability of Activity Patterns measured with Accelerometers in Chronic Pain Patients
Methods

### Data collection

### Data processing

Model description

### Statistics

After the conversion of the accelerometer data into physical-intensity levels per minute, 50 characteristics describing the activity patterns were calculated. The characteristics with formulas and description are described in table 1.

Correlation

Collinearity among characteristics was assessed using Pearson’s correlation coefficient; factors showing overlap were removed to reduce redundancy. If two characteristics showed a correlation coefficient ≥ .85, the most complicated characteristics was removed.

Reliability

The inter-day reliability of all characteristics was calculated using the intraclass correlation coefficient (ICC 2.1) [ref]. An ICC of .5 - .75 was seen as moderate reliability, .75 - .9 as good and ≥ .9 as excellent [ref]. Additionally, the confidence interval (CI), standard error of measurement (SEM) and the minimal detectable change (MDC) were calculated. The MDC represent the threshold in which changes in score exceed measurement error [ref]. To identify the minimal number of days needed to achieve reliable results, reliability was assessed for measurement durations of one to six days, using repeated measurements on the corresponding days of the subsequent week. The reliability was calculated for both CP and healthy controls to compare outcomes?

Comparison CP and healthy controls

Outcomes were compared?

Table 1. Characteristics of activity patterns and corresponding formulas and description

| Characteristic | Formula | Description | Meaning |
| --- | --- | --- | --- |
| x̄ intensity | x̄ | Average activity level (0 sedentairy, 1 light intensity, 2 moderate-vigorous intensity) | indication of someone's average activity over the day |
| intensity |  | Standard deviation of the activity levels | indication of the variation of activities during the day |
| intensity | / | Number of transitions devided by total number of samples | indication of how often someone changes is activity during the day |
| Transitions X -> Y | Transition | Count value of how often an individual transitions from | indication of how often someone transitions from one specific activity class to another |
| activity level |  | count value of how often a specific activity level is registered | indication of someone's most common activity level |
| % activity level |  | Relative percentage value of how often a specific activity level occurs. | Indication of someone's most common activity level |
| Alfa |  |  |  |
| Sigma |  |  |  |
| Gini |  |  |  |
| x̄ epoch length |  | average ebook length | indication of how long someone does stays in a specific activity on average |
| Max epoch length |  | maximum epoch length | indication of the Max time that someone spends in a specific activity level |
| Weighted median epoch length |  | the value where the sum of weights of all data pointsis split in half |  |
| Sample entropy |  |  | indication of the complexity of the movement patterns |
| Information entropy |  |  | indication of the complexity of the movement patterns |
| Permutation Lempel-Ziv complexity |  |  | indication of the complexity of the movement patterns |
| Definitions: Activiti level = ; epoch = ; movement patterns = ; |  |  |  |

# Results

### Characteristics

Data were collected from 40 individuals with chronic pain (CP) and 27 healthy controls. For the healthy control group, data from a total of 356 days were recorded. Of these, 20 days were excluded due to insufficient duration (<10 hours) or excessive duration (>20 hours). The remaining recordings comprised 4,735 hours of valid data, averaging 13.3 hours per day. On average, 52.1% of this time was spent sedentary, 40.4% in light-intensity activity, and 7.5% in moderate-intensity activity. For the chronic pain group, data from a total of 685 days were collected, of which 51 days were excluded for being too short or too long. The remaining valid data covered 8,829 hours in total, with 53.8% of time spent sedentary, 39.6% in light-intensity activity, and 6.6% in moderate-intensity activity.

Table 2. Patient Characteristics

### Correlation coefficient

The correlation coefficients were calculated using data from both the CP and the healthy controls. 27 of the 50 calculated characteristics were excluded due to an correlation coefficient higher than 0.85. These include the Permutation Lempel-Ziv complexity, Sample entropy which were highly correlated with the  intensity. The complete table with all the correlation coefficients is reported in table S1 in the supplements.

### Reliability

Data of 34 CP and 22 healthy controls contained at least 12 valid measurement days. The test-retest reliability is evaluated by comparing the average outcome of N days of the first week to the average outcome of N days in the second week using the ICC values. The outcomes for the 27 characteristics for the CP are reported in Table 3. The outcomes for the healthy controls is reported in table 4.

Table 3. ICC values of CP per day.

|  |  | 1 day | 2 days | 3 days | 4 days | 5 days | 6 days |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | x̄ intensity | 0.74 | 0.74 | 0.75 | 0.72 | 0.78 | 0.71 |
|  | intensity | 0.69 | 0.84 | 0.88 | 0.83 | 0.83 | 0.78 |
|  | Transitions 0 - 1 | 0.64 | 0.83 | 0.88 | 0.85 | 0.89 | 0.87 |
|  | Transitions 0 -2 | 0.22 | 0.51 | 0.54 | 0.66 | 0.66 | 0.70 |
|  | Transitions 2 - 0 | 0.33 | 0.51 | 0.62 | 0.67 | 0.71 | 0.62 |
|  | Information entropy | 0.53 | 0.54 | 0.59 | 0.65 | 0.69 | 0.77 |
| Sedentary | % | 0.65 | 0.72 | 0.72 | 0.72 | 0.76 | 0.71 |
|  | Weighted median epochs | 0.64 | 0.84 | 0.83 | 0.58 | 0.70 | 0.61 |
|  | Alfa | 0.44 | 0.49 | 0.52 | 0.51 | 0.67 | 0.55 |
|  | Sigma | 0.57 | 0.57 | 0.57 | 0.51 | 0.64 | 0.43 |
|  | Gini | 0.59 | 0.64 | 0.68 | 0.70 | 0.74 | 0.69 |
|  | x̄ epoch length | 0.55 | 0.74 | 0.82 | 0.78 | 0.85 | 0.80 |
|  | Max epoch length | 0.53 | 0.74 | 0.73 | 0.62 | 0.66 | 0.59 |
| Light intensity | % | 0.67 | 0.70 | 0.67 | 0.66 | 0.76 | 0.67 |
|  | Weighted median epochs | 0.63 | 0.65 | 0.59 | 0.71 | 0.73 | 0.71 |
|  | Alfa | 0.61 | 0.74 | 0.68 | 0.75 | 0.77 | 0.68 |
|  | Sigma | 0.61 | 0.72 | 0.66 | 0.71 | 0.78 | 0.67 |
|  | Ginie | 0.51 | 0.68 | 0.63 | 0.74 | 0.74 | 0.71 |
|  | x̄ epoch length | 0.70 | 0.73 | 0.67 | 0.77 | 0.78 | 0.72 |
|  | Max epoch length | 0.23 | 0.43 | 0.43 | 0.58 | 0.62 | 0.61 |
| Moderate Intensity | % | 0.73 | 0.73 | 0.79 | 0.81 | 0.78 | 0.80 |
|  | Weighted median epochs | 0.43 | 0.26 | 0.62 | 0.84 | 0.83 | 0.81 |
|  | Alfa | 0.25 | 0.34 | 0.27 | 0.17 | 0.29 | 0.30 |
|  | Sigma | 0.18 | 0.37 | 0.32 | 0.23 | 0.33 | 0.37 |
|  | Gini | 0.47 | 0.55 | 0.68 | 0.77 | 0.79 | 0.83 |
|  | x̄ epoch length | 0.75 | 0.62 | 0.74 | 0.80 | 0.78 | 0.80 |
|  | Max epoch length | 0.77 | 0.77 | 0.79 | 0.80 | 0.80 | 0.81 |
| Total averages per day |  | 0.54 | 0.63 | 0.65 | 0.67 | 0.71 | 0.68 |
| Abbreviations: Description: Variables with good-excellent reliability ( 0.75) were marked in bold. |  |  |  |  |  |  |  |

Table 4. ICC values of healthy controls per day.

|  |  | 1 day | 2 days | 3 days | 4 days | 5 days | 6 days |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | x̄ intensity | 0.63 | 0.66 | 0.72 | 0.76 | 0.73 | 0.79 |
|  | intensity | 0.67 | 0.78 | 0.85 | 0.80 | 0.78 | 0.77 |
|  | Transitions 0 - 1 | 0.40 | 0.61 | 0.79 | 0.77 | 0.76 | 0.74 |
|  | Transitions 0 -2 | 0.43 | 0.54 | 0.67 | 0.51 | 0.50 | 0.68 |
|  | Transitions 2 - 0 | 0.26 | 0.59 | 0.76 | 0.69 | 0.74 | 0.81 |
|  | Information entropy | 0.36 | 0.45 | 0.63 | 0.54 | 0.73 | 0.81 |
| Sedentary | % | 0.38 | 0.50 | 0.79 | 0.77 | 0.84 | 0.87 |
|  | Weighted median epochs | 0.72 | 0.68 | 0.74 | 0.69 | 0.63 | 0.71 |
|  | Alfa | 0.62 | 0.77 | 0.70 | 0.76 | 0.68 | 0.69 |
|  | Sigma | 0.57 | 0.77 | 0.70 | 0.78 | 0.71 | 0.71 |
|  | Gini | 0.42 | 0.54 | 0.57 | 0.66 | 0.59 | 0.65 |
|  | x̄ epoch length | 0.61 | 0.74 | 0.77 | 0.73 | 0.66 | 0.70 |
|  | Max epoch length | 0.48 | 0.64 | 0.71 | 0.75 | 0.60 | 0.69 |
| Light intensity | % | 0.53 | 0.59 | 0.58 | 0.66 | 0.71 | 0.78 |
|  | Weighted median epochs | 0.30 | 0.47 | 0.56 | 0.66 | 0.77 | 0.84 |
|  | Alfa | 0.25 | 0.33 | 0.36 | 0.67 | 0.74 | 0.85 |
|  | Sigma | 0.36 | 0.26 | 0.31 | 0.61 | 0.67 | 0.79 |
|  | Ginie | 0.32 | 0.46 | 0.44 | 0.54 | 0.70 | 0.71 |
|  | x̄ epoch length | 0.27 | 0.40 | 0.51 | 0.66 | 0.81 | 0.89 |
|  | Max epoch length | 0.02 | 0.13 | 0.18 | 0.39 | 0.50 | 0.61 |
| Moderate Intensity | % | 0.62 | 0.66 | 0.70 | 0.76 | 0.75 | 0.81 |
|  | Weighted median epochs | 0.32 | 0.58 | 0.64 | 0.76 | 0.79 | 0.80 |
|  | Alfa | 0.38 | 0.54 | 0.63 | 0.71 | 0.42 | 0.34 |
|  | Sigma | 0.38 | 0.60 | 0.70 | 0.78 | 0.45 | 0.37 |
|  | Gini | 0.54 | 0.69 | 0.79 | 0.81 | 0.81 | 0.80 |
|  | x̄ epoch length | 0.38 | 0.64 | 0.76 | 0.78 | 0.81 | 0.76 |
|  | Max epoch length | 0.36 | 0.56 | 0.63 | 0.69 | 0.75 | 0.72 |
| Total averages per day |  | 0.43 | 0.56 | 0.64 | 0.69 | 0.69 | 0.73 |
| Abbreviations: Description: Variables with good-excellent reliability ( 0.75) were marked in bold. |  |  |  |  |  |  |  |